My Network Configuration

R1-SITE1#wr mem

Building configuration...

[OK]

R1-SITE1#

R1-SITE1#show running-config

Building configuration...

Current configuration : 1706 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname R1-SITE1

!

!

!

enable secret 5 $1$mERr$GNyuTtIItAetqnRcuf4kz/

!

!

ip dhcp excluded-address 192.168.0.1 192.168.0.10

ip dhcp excluded-address 192.168.0.200 192.168.0.254

ip dhcp excluded-address 192.168.128.1

!

ip dhcp pool DHCP_SITE1

network 192.168.0.0 255.255.255.0

default-router 192.168.0.1

dns-server 8.8.8.8

ip dhcp pool DEV_POOL

network 192.168.128.0 255.255.255.128

default-router 192.168.128.1

dns-server 8.8.8.8

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2911/K9 sn FTX1524W5CP-

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

ip address 216.127.251.2 255.255.255.252

ip nat outside

duplex auto

speed auto

!

interface GigabitEthernet0/1

description SITE1_LAN

ip address 192.168.0.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/1.1

description Gateway for VLAN 1

encapsulation dot1Q 1 native

no ip address

ip nat inside

!

interface GigabitEthernet0/1.20

description Gateway for VLAN 20 (DEV)

encapsulation dot1Q 20

ip address 192.168.128.1 255.255.255.128

ip nat inside

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Vlan1

no ip address

shutdown

!

ip nat inside source list 1 interface GigabitEthernet0/0 overload

ip classless

ip route 0.0.0.0 0.0.0.0 216.127.251.1

!

ip flow-export version 9

!

!

access-list 1 permit 192.168.0.0 0.0.0.255

access-list 1 permit 192.168.128.0 0.0.0.127

!

!

!

!

!

line con 0

password 7 08376C4200180B03

login

!

line aux 0

!

line vty 0 4

password 7 0802787C584B56

login

!

!

!

End

R2-SITE2#show running-config

Building configuration...

Current configuration : 952 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname R2-SITE2

!

!

!

enable secret 5 $1$mERr$GNyuTtIItAetqnRcuf4kz/

!

!

!

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2911/K9 sn FTX1524UXD7-

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

ip address 216.127.251.10 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/1

ip address 192.168.1.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

log-adjacency-changes

passive-interface GigabitEthernet0/1

network 0.0.0.0 255.255.255.255 area 0

!

ip classless

!

ip flow-export version 9

!

!

!

!

!

!

!

line con 0

password 7 08376C4200180B03

login

!

line aux 0

!

line vty 0 4

password 7 0802787C584B56

login

!

!

!

End

S1-SITE1>en

Password:

S1-SITE1#wr mem

Building configuration...

[OK]

S1-SITE1#

S1-SITE1#show run

Building configuration...

Current configuration : 1325 bytes

!

version 12.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname S1-SITE1

!

enable secret 5 $1$mERr$GNyuTtIItAetqnRcuf4kz/

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

spanning-tree vlan 1,20 priority 4096

!

interface Port-channel1

!

interface Port-channel2

!

interface FastEthernet0/1

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

channel-group 2 mode active

!

interface FastEthernet0/24

channel-group 2 mode active

!

interface Vlan1

ip address 192.168.0.201 255.255.255.0

!

ip default-gateway 192.168.0.1

!

!

!

!

line con 0

password 7 08376C4200180B03

login

!

line vty 0 4

password 7 0802787C584B56

login

line vty 5 15

login

!

!

!

!

End

S2-SITE1#show running-config

Building configuration...

Current configuration : 1298 bytes

!

version 12.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname S2-SITE1

!

enable secret 5 $1$mERr$GNyuTtIItAetqnRcuf4kz/

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface Port-channel1

switchport mode dynamic desirable

!

interface FastEthernet0/1

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

channel-group 1 mode passive

!

interface FastEthernet0/24

channel-group 1 mode passive

!

interface Vlan1

ip address 192.168.0.202 255.255.255.0

!

ip default-gateway 192.168.0.1

!

!

!

!

line con 0

password 7 08376C4200180B03

login

!

line vty 0 4

password 7 0802787C584B56

login

line vty 5 15

login

!

!

!

!

end

S3-SITE1#show run

Building configuration...

Current configuration : 1349 bytes

!

version 12.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname S3-SITE1

!

enable secret 5 $1$mERr$GNyuTtIItAetqnRcuf4kz/

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface Port-channel2

switchport mode dynamic desirable

!

interface FastEthernet0/1

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

channel-group 2 mode passive

!

interface FastEthernet0/24

channel-group 2 mode passive

!

interface Vlan1

ip address 192.168.0.203 255.255.255.0

!

ip default-gateway 192.168.0.1

!

!

!

!

line con 0

password 7 08376C4200180B03

login

!

line vty 0 4

password 7 0802787C584B56

login

line vty 5 15

login

!

!

!

!

end