# Creating and Applying an ACL - Worksheet

In this project, you will create an ACL for VLAN 30 to accomplish the tasks given in the project. This project can be completed is as little as 1 minute. It is not expected to take longer than 30. There is an opportunity for 10 bonus points in this project.

Here is the information you will need for the project:

IPv4 Assignments:

| PC Name | VLAN Name | VLAN Configuration |
| --- | --- | --- |
| Sales 1 | Sales | 192.168.1.10 |
| Marketing 1 | Marketing | 192.168.2.10 |
| Admin 1 | Admin | 192.168.3.10 |
| Sales 2 | Sales | 192.168.1.20 |
| Marketing 2 | Marketing | 192.168.2.20 |
| Admin 2 | Admin | 192.168.2.20 |
| Sales 3 | Sales | 192.168.3.10 |
| Marketing 3 | Marketing | 192.168.3.20 |
| Admin 3 | Admin | 192.168.3.30 |
| Provo DNS |  | 172.16.0.6 |
| Provo Net Admin |  | 172.16.0.5 |
| www.company.internal |  | 172.168.0.3 |
| www.ralph.com |  | 198.168.5.2 |

VLAN Information:

| VLAN Information | Port Assignments |
| --- | --- |
| 1 Management | 13-16 |
| 10 Sales | 1 – 4 |
| 20 Marketing | 5 - 8 |
| 30 Admin | 9 – 12 |

Provo Router:

| Interface | VLAN | IP Address | Encapsulation |
| --- | --- | --- | --- |
| gi0/0 | N/A | no ip address | none |
| gi0/0.1 | Sales | 192.168.1.1 | dot1q |
| gi0/0.2 | Marketing | 192.168.2.1 | dot1q |
| gi0/0.3 | Admin | 192.168.3.1 | dot1q |
| gi0/0.4 | VLAN 1 | 192.168.4.1 | dot1q |
| gi1/0 |  | 172.16.0.1 |  |
| s2/0 |  | 198.3.24.4 |  |

|  | Points Possible | Points Awarded |
| --- | --- | --- |
| - Open Creating_and_Applying_an_ACL_Student_Project.pkt. - Open Admin 3 and verify that you can ping: - Simulated Internet (198.168.5.2) - Provo Network Admin (172.16.0.5) - Sales 2 (192.168.1.20) - Marketing 1 (192.168.2.10) |  |  |
|  | 1 |  |
| - Open the web browser and access www.company.internal |  |  |
|  | 1 |  |
| - Open the Provo router and create ACL 130. Take a snip of the created ACL (show access-list) |  |  |
|  | 2 |  |
| - Apply the ACL to the appropriate interface and direction. Take a snip of the running configuration showing the ACL applied to the interface. |  |  |
|  | 2 |  |
| - Attempt to ping the Internet (198.168.5.2). It should fail - Attempt to ping the Provo DNS (172.16.0.6). It should succeed - Post snip of the ping results below. |  |  |
|  | 1 |  |
| - Attempt to ping Sales 1 (192.168.1.10). It should fail - Attempt to ping Admin 1 (192.168.3.10). It should succeed - Paste the ping results below. |  |  |
|  | 1 |  |
| - Open the Admin 3 Web Browser and access www.ralph.com. Paste results below. |  |  |
|  | 2 |  |
| - BONUS! Open a ssh session between Provo Network Admin and the Team3 switch. |  |  |
|  | 10 |  |
| Points | 10 |  |
| Bonus Points | 10 |  |
| Total Points | 20 |  |